| Ousmane Toure & Jianning Chen EECE2160 | Embedded Design: Enabling Robotics Prelab Assignment 7 |
| --- | --- |

Prelab Assignment 7

Ousmane Toure & Jianning Chen

Toure.o@northeastern.edu

chen.jiann@northeastern.edu

Submit date: 7/14/2022

Due Date: 7/14/2022

7.0.1 Hexadecimal digits to display on the 7-segment displays

Table 7.1

| Character | OUTPUTS - Segment 0/1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | 6 | 5 | 4 | 3 | 2 | 1 | 0 | Decimal | Hex |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 63 | 0x3F |
| 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 6 | 0x06 |
| 2 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 91 | 0x5B |
| 3 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 79 | 0x4F |
| 4 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 102 | 0x66 |
| 5 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 109 | 0x6D |
| 6 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 125 | 0x7D |
| 7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 7 | 0x07 |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 127 | 0x7F |
| 9 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 111 | 0x6F |
| A | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 119 | 0x77 |
| B | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 124 | 0x7C |
| C | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 57 | 0x39 |
| D | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 94 | 0x5E |
| E | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 121 | 0x79 |
| F | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 113 | 0x71 |

7.0.2 Where is figure 1 found?

Figure 1 is found on page 8 of DE1-SoC_Computer_System_with_ARM_Cortex_A9.pdf.

7.0.3

Picture of number input 1551231:

Picture of number input 255:

Picture of number 10:

Code:

#include <iostream>

#include "SevenSegment.h"

#include "DE1SoCfpga.h"

SevenSegment::SevenSegment(){

reg0_hexValue = (bit_values[0] << 24) | (bit_values[0] << 16) | (bit_values[0] << 8) | bit_values[0];

reg1_hexValue = (bit_values[0] << 8) | bit_values[0];

RegisterWrite(HEX3_0_OFFSET,reg0_hexValue);

RegisterWrite(HEX5_4_OFFSET,reg1_hexValue);

}

SevenSegment::~SevenSegment() {

Hex_ClearAll();

}

void SevenSegment::Hex_ClearAll() {

reg0_hexValue = 0x3F3F3F3F;

reg1_hexValue = 0x3F3F3F3F;

RegisterWrite(HEX3_0_OFFSET,reg0_hexValue);

RegisterWrite(HEX5_4_OFFSET,reg1_hexValue);

}

void SevenSegment::Hex_ClearSpecific(int index) {

if(index == 0){reg0_hexValue = reg0_hexValue & 0x00F;}

else if(index == 1){reg0_hexValue = reg0_hexValue & 0x0F0;}

else if(index == 2){reg0_hexValue = reg0_hexValue & 0xF00;}

else if(index == 3){reg0_hexValue = reg0_hexValue & 0xF000;}

else if(index == 4){reg1_hexValue = reg1_hexValue & 0x00F;}

else if(index == 5){reg1_hexValue = reg1_hexValue & 0x0F0;}

else{

std::cerr << "Index exceeding 5 or less than 0!" << std::endl;

exit(1);

}

}

void SevenSegment::Hex_WriteNumber(int number) {

int temp;

temp = number;

int store[6] = {0,0,0,0,0,0};

int remainder;

int count = 0;

while(temp > 0){

remainder = temp%16;

store[count] = remainder;

std::cout << remainder;

++count;

temp = temp/16;

}

reg0_hexValue = 0x00;

reg1_hexValue = 0x00;

reg0_hexValue = (bit_values[store[3]] << 24) | (bit_values[store[2]] << 16) | (bit_values[store[1]] << 8) | bit_values[store[0]];

reg1_hexValue = (bit_values[store[5]] << 8) | bit_values[store[4]];

RegisterWrite(HEX3_0_OFFSET,reg0_hexValue);

RegisterWrite(HEX5_4_OFFSET,reg1_hexValue);

}

void SevenSegment::Hex_WriteSpecific(int index, int value) {

int temp;

if(value > 15 || value < -16) {

std::cerr << "Value exceeding boundary!" << std::endl;

exit(1);

}

else if(value >= 0) {temp = value;}

else{temp = 16+value;}

temp = value;

if(index < 4){reg0_hexValue = reg0_hexValue | (bit_values[temp] << 8*index);}

else{reg1_hexValue = reg1_hexValue | (bit_values[temp] << 8*(index-4));}

RegisterWrite(HEX3_0_OFFSET,reg0_hexValue);

RegisterWrite(HEX5_4_OFFSET,reg1_hexValue);

}

/*

int SevenSegment::PushButtonGet() {

int value = this->RegisterRead(pBase,KEY_OFFSET);

return value;

}

*/

DE1SoCfpga::DE1SoCfpga(){

fd = open("/dev/mem",(O_RDWR | O_SYNC));

if(fd == -1){

std::cout << "ERROR: could not open /dev/mem..." << std::endl;

exit(1);

}

char *virtual_base = (char *)mmap(NULL, LW_BRIDGE_SPAN,(PROT_READ | PROT_WRITE), MAP_SHARED, fd, LW_BRIDGE_BASE);

if(virtual_base == MAP_FAILED){

std::cout << "ERROR: mmap() failed..." << std::endl;

close(fd);

exit(1);

}

pBase = virtual_base;

}

DE1SoCfpga::~DE1SoCfpga() {

if(munmap(pBase,LW_BRIDGE_SPAN) != 0){

std::cout << "ERROR: munmap() failed..." << std::endl;

exit(1);

}

close(fd);

}

//int DE1SoCfpga::RegisterRead(unsigned int offset) {

//return *(volatile unsigned int *)(pBase + offset);

//}

void DE1SoCfpga::RegisterWrite(unsigned int offset, unsigned int value) {

*(volatile unsigned int *)(pBase + offset) = value;

}

int main()

{

SevenSegment *display = new SevenSegment;

std::cout << "Program Starting...!" << std::endl;

bool condition = true;

int hex_value;

while(condition){

std::cout << "Please enter a number for display (in the range -16777216 to 16777215):";

std::cin >> hex_value;

display->Hex_WriteNumber(hex_value);

std::cout << "Do you want to stop? ";

char v;

std::cin >> v;

if(v == 'y'){condition = false;}

}

std::cout << "Terminating...!" << std::endl;

delete display;

return 0;

}

# References

- Prof. Julius Marpaung, “Lab Report Guide”, Northeastern University, January 6 2020.